# An Example 8051 Microprocessor Program

Here is an example of what a program for the 8051 microprocessor looks like. This particular program employs the serial port on the 8051 microprocessor which is an input/output device exactly like the serial port on a PC. Every time the human operator types a character on the PC's keyboard, the 8051 simulator places the corresponding ASCII byte in the 8051 microprocessor's serial port register SBUF. And each time a byte is received over the serial port, an 8051 hardware interrupt occurs, causing execution to branch to a predefined interrupt service vector. In this example program, we have chosen to place assembly code at this interrupt service vector that causes the keyboard letter to be displayed (scrolled) onto 8 consecutive columns of the electronic sign board. This particular program consists of only 42 statements and its operation is fully explained in the on-line Help documentation.

; This program paints on the signboard whatever characters the operator

; types on the computer's keyboard.  The 8051 simulator creates a

; "serial port interrupt" each time the operator types an alphabetic

; character.  Serial port interrupts are 1 of the 5 types of interrupts

; that an 8051 microprocessor can respond to.  Once you have written

; to the IE ("interrupt enable") register, the response to an interrupt

; is automatic and occurs asynchronously to whatever other activity

; the 8051 program is involved in.  The response to an interrupt is

; exactly as if the program had performed an LCALL instruction.  That

; is, program execution jumps off to an interrupt service routine that

; you have placed at a predefined location in program memory.  When

; Intel designed the 8051 the locations for these 5 interrupt service

; routines was permanently fixed.  The serial port interrupt that

; this program employs begins at program memory address 0x23.  When

; we reach the RETI instruction at the end of the interrupt service

; routine, the 8051 goes back to doing whatever it had been doing

; before it was interrupted.

ORG   0x00       ; this statement is not really necessary as it

;   is the default behavior of the assembler

LJMP  Start      ; jump over the prog mem locations reserved for ISVs

;   (interrupt service vectors) and jump over

;   the non-executable DB statements brought in

;   by the inclusion of the FontData.txt file.

ORG   0x03       ; the ISV for the EXT 0 interrupt is 0x0003

EXT0INT: LJMP  Int0Isr

ORG   0x0B       ; the ISV for the TMR 0 interrupt is 0x000B

TMR0INT: LJMP  Tmr0Isr

ORG   0x13       ; the ISV for the EXT 1 interrupt is 0x0013

EXT1INT: LJMP  Int1Isr

ORG   0x1B       ; the ISV for the TMR 1 interrupt is 0x001B

TMR1INT: LJMP  Tmr1Isr

ORG   0x23       ; the ISV for the Serial Port interrupt is 0x0023

SERINT:  LJMP  SerIsr

INCLUDE  "FontData.txt"

ORG   0x100      ; the interrupt service vectors consume program memory

;   locations 0x03 thru 0x25 = 37 and the DB statements

;   in FontData.txt consume another 27*8 = 216 bytes

;   (remember there is a space character after Z).

;   Hence this program memory address of 0x100 = 256 is

;   safely past these items.

Start:   MOV   DPTR,#StartOfImageData

; We initialize the DPTR ("data pointer"), a 16 bit

;   SFR ("special function register"), to point to

;   the first column of font data (the first column

;   of the letter 'A') in program memory.  The DPTR

;   register continues to hold this same value

;   throughout this program.

MOV   SCON,#0x10 ; assert REN so the serial port is enabled to receive

MOV   IE,#0x90   ; 10010000B, MSBit is the global intr enable,

;   the other bit we raise is ES, the serial

;   port interrupt enable

MOV   A, #0x01   ; we walk this 1 bit across the signboard while

;   waiting for an interrupt

Loop:    RL    A          ; loop while waiting for an interrupt

MOV   P0,A

SJMP  Loop

; If we somehow land in any of the interrupt service routines that this

; program is not prepared to handle we just initiate an infinite loop

; so we can detect the mistake.

Int0Isr:                  ; interrupt service routine for EXT 0 interrupt

SJMP  Int0Isr

Int1Isr:                  ; interrupt service routine for EXT 1 interrupt

SJMP  Int1Isr

Tmr0Isr:                  ; interrupt service routine for TMR 0 interrupt

SJMP  Tmr0Isr

Tmr1Isr:                  ; interrupt service routine for TMR 1 interrupt

SJMP  Tmr1Isr

SerIsr:                   ; interrupt service routine for the serial port

;  interrupt

PUSH  ACC        ; protect the prior value in the ACC since the

;  interrupt service routine needs to use it

MOV   A,SBUF     ; read alphabet index from serial buffer SFR

LCALL Draw1Char

POP   ACC        ; restore the prior value of the ACC

MOV   SCON,#0x10 ; clear the RI bit in SCON (leave REN asserted)

RETI

Draw1Char:

; This is a subroutine which is responsible for outputting exactly 8

; columns (hence 1 character) of font data to the electric sign board.

; Before you call this subroutine initialize the ACC with the index

; of the desired character.  That is, if you want the letter 'C'

; output to the sign board then call this subroutine with ACC = 2.

; We first need to convert the letter index (ranging from 0 to 26)

; to a column index (ranging from 0 to 208).  This column index

; will describe the byte offset to that letter's column data.

; the ACC presently holds an alphabet index

MOV   B,#8     ; each letter has 8 columns

MUL   AB       ; This computes the byte offset beyond

;   StartOfImageData where the letter starts. This

;   product is guaranteed to be < 256 since the

;   entire alphabet only has 216 (27*8) columns

;   (remember there is a space char after Z),

;   hence we only need to keep (deal with) the

;   LSByte of the product.  The LSByte is found

;   in the ACC following the "MUL AB" instr.

; Within this subroutine we use R0 as a temporary storage location

; for the accumulator's value (we need this because the "MOVC A,@A+DPTR"

; instruction keeps perturbing the accumulator).  And we use the

; R1 register to control the loop (that is, decide how many times

; we iterate).  Note that R0 is synonymous with data memory location 0

; and R1 is synonymous with data memory location 1 (at least while the

; 8051 is in its default configuration).

MOV   R0,A     ; preserve this starting column # since the

;   upcoming "MOVC A,@A+DPTR" instruction

;   replaces the value in the ACC

MOV   R1,#0    ; we use R1 to control the looping

CharLoop:  MOV  A,R0      ; prepare for upcoming "MOVC A,@A+DPTR" instruction

MOVC A,@A+DPTR ; the ACC now holds one column of font data

MOV  P0,A      ; write that column to the sign board

INC  R0        ; advance to the next column

INC  R1        ; increment the loop counter

CJNE R1,#8,CharLoop

; we loop 8 times in order to output 8 columns

RET